DeMoD LLC

ACOUSTIC FSK

MODEM GUIDE

Two-tone audible FSK over speaker + microphone. Sounds like dial-up. That is the point.

v0.3.0  ·  GPL-3.0  ·  info@demod.ltd  ·  JackFrame 8-byte protocol

| MODULATION 2-tone FSK (AFSK) Mark = 2000 Hz Space = 3000 Hz h = 0.833 | TIMING 1200 baud 48 kHz SR sps = 40 (integer) Frame = 53.3 ms | FRAME JackFrame 8 bytes 4-byte payload CRC-CCITT pin 0xC23F sync byte = 0xA5 |
| --- | --- | --- |

# 1  How It Works

This is a software-defined audio modem. The transmitting machine encodes data as two audio tones — 2000 Hz for a 1-bit, 3000 Hz for a 0-bit — and plays them through its speaker. The receiving machine captures those tones with its microphone and decodes them back to bytes.

The tones are audible. At 1200 baud they produce a recognisable warbling that sounds exactly like a dial-up modem handshake. This is intentional.

## 1.1  Signal chain

| JackFrame 8 bytes | → | NRZ-L bits ±1 floats | → | Faust CP-FSK phasor | → | Speaker audio | ~air~ | Mic capture | → | Dual BPF decoder |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 1.2  Why FSK, not BPSK

The guitar-cable variant of this modem uses BPSK with a Costas loop. That cannot work over air. Room reflections arrive 5–7 symbol periods late at 1200 baud — seven ghost copies of the signal overlap and destroy phase coherence. FSK sidesteps this entirely: the demodulator compares energy in two frequency bands, which is robust to amplitude distortion from multipath. No phase reference, no Costas loop, no carrier sync required.

## 1.3  Demodulator algorithm

The receiver uses a dual-filter energy detector — two resonant bandpass filters centred at 2000 Hz and 3000 Hz, followed by envelope followers. The decision is the difference in energy between the two filters: positive means the mark tone (2000 Hz) is louder, so the bit is 1. Negative means the space tone (3000 Hz) is louder, so the bit is 0. This differential form rejects common-mode noise because both filters see the same ambient room sound.

# 2  Hardware

This modem is designed to run on whatever audio hardware is available. Laptop built-ins work. External speakers and USB microphones work better. The key constraint is the 1400–3600 Hz signal band — almost any speaker or microphone covers this range.

| PARAMETER | VALUE / NOTES |
| --- | --- |
| Laptop speakers + mic | Works at 0.3–0.5m (same desk). AGC in the mic preamp may cause initial dropouts. |
| USB desktop mic (e.g. Blue Yeti) | Better SNR, more consistent gain, recommended for reliable RX. |
| Phone speaker → laptop mic | Surprisingly works. Phone speaker is flat through 4 kHz. |
| External speakers + USB mic | Best combination. Most predictable frequency response. |
| Guitar interface (Scarlett, UMC) | Overkill but works perfectly. Enables the TS-cable variant too. |

| ! | DISTANCE Keep TX speaker and RX mic within 0.5m for reliable decoding. At 1m, room reflections add enough multipath to cause occasional bit errors. At 2m+, expect significant packet loss without an equaliser. |
| --- | --- |

| ! | FEEDBACK On the RX machine, mute the system output or use headphones while the mic is open. Speaker → mic feedback at 2–3 kHz will saturate the input and prevent decoding. |
| --- | --- |

# 3  Build

## 3.1  Faust DSP compilation

The modulator and demodulator each require one Faust compile step before cabal build. Run these from inside the Nix dev shell.

TERMINAL — Faust compile

# Enter the Haskell nix dev shell from the repo root: cd haskell && nix develop # Compile the acoustic FSK modulator: faust -a ../arch/soapy-sdr-lib.cpp -lang c++ -vec -vs 256 \ dsp/acoustic_fsk_mod.dsp -o build/acoustic_fsk_mod_gen.cpp # Compile the acoustic FSK demodulator: faust -a ../arch/soapy-sdr-lib.cpp -lang c++ -vec -vs 256 \ dsp/acoustic_fsk_demod.dsp -o build/acoustic_fsk_demod_gen.cpp # Or use the shell aliases (add to flake.nix): faust-acoustic-mod faust-acoustic-demod

## 3.2  Add to cabal file

dcf-faust-sdr.cabal

-- In dcf-faust-sdr.cabal, add to library exposed-modules: DCF.Modem.AcousticFSK DCF.Transport.JackFrame -- Add executables: executable acoustic-tx main-is: AcousticTx.hs hs-source-dirs: app build-depends: base, dcf-faust-sdr, vector, bytestring executable acoustic-rx main-is: AcousticRx.hs hs-source-dirs: app build-depends: base, dcf-faust-sdr, vector, bytestring

## 3.3  Build and verify

TERMINAL

cabal build all # Verify the JackFrame CRC pin: python3 -c " d=bytes([0xA5,0x01,0xDE,0xAD,0xBE,0xEF]) crc=0xFFFF for b in d: crc^=b<<8 for _ in range(8): crc=((crc<<1)^0x1021)if crc&0x8000 else crc<<1 crc&=0xFFFF assert crc==0xC23F, hex(crc) print("JackFrame CRC pin OK: 0xC23F") "

# 4  Audio Routing (JACK / PipeWire)

The acoustic modem requires real-time audio I/O with minimal buffering. On ArchibaldOS (CachyOS RT kernel + PipeWire), this is automatic. On a standard Linux system, use JACK2 or configure PipeWire for low-latency operation.

## 4.1  PipeWire (ArchibaldOS / NixOS with PipeWire)

TERMINAL — PipeWire

# Check PipeWire is running: systemctl --user status pipewire # Set audio output to your speaker device: pactl list sinks short # find your speaker sink name pactl set-default-sink <sink-name> # Set audio input to your microphone: pactl list sources short # find your mic source name pactl set-default-source <source-name> # Check buffer size (lower = less latency, more CPU): pw-metadata -n settings ¦ grep quantum # Set to 256 samples for a good TX/RX balance: pw-metadata -n settings 0 clock.force-quantum 256

## 4.2  JACK2

TERMINAL — JACK2

# Start JACK with 256-sample buffer at 48kHz: jackd -d alsa -r 48000 -p 256 -n 2 & # Connect acoustic-tx output to system playback (speaker): jack_connect acoustic-tx:output_0 system:playback_1 # Connect system capture (mic) to acoustic-rx input: jack_connect system:capture_1 acoustic-rx:input_0 # Or use QjackCtl for a visual patchbay.

| i | BUFFER SIZE 256 samples at 48kHz = 5.33ms buffer latency. For the demo this is fine — frame airtime (53.3ms) dominates. If you get xruns, increase to 512. If you want lower total latency, decrease to 128 (2.67ms) — this works on the RT kernel, may xrun on a stock kernel. |
| --- | --- |

# 5  Running the Demo

Set up two machines on the same desk, speaker facing the other machine's mic, within 30–50 cm. Run acoustic-tx on one machine and acoustic-rx on the other.

## 5.1  Physical setup

| Machine A (TX) Speaker facing → System output unmuted Mic muted (or headphones) | Machine B (RX) ← Mic facing speaker Mic unmuted, gain ~50–70% System output muted |
| --- | --- |

Distance: 30–50 cm is the sweet spot. Too close and mic saturation can cause clipping. Too far and SNR drops below the demodulator threshold. Start at 40 cm and adjust.

## 5.2  Run RX first (Machine B)

Always start the receiver before the transmitter. The demodulator needs a few frames to let the mic AGC settle before reliable decoding begins.

TERMINAL — Machine B

# Machine B — start RX, listen for frames: cabal run acoustic-rx -- \ --device default \ --rate 48000 \ --buffer 256 # Expected output: # [acoustic-rx] Waiting for mic AGC to settle... # [acoustic-rx] Listening for frames... # (silence until TX starts)

## 5.3  Run TX (Machine A)

TERMINAL — Machine A

# Machine A — transmit beacon frames with incrementing counter: cabal run acoustic-tx -- \ --device default \ --rate 48000 \ --buffer 256 \ --count 20 # send 20 frames then stop (omit for infinite) # Expected output: # [acoustic-tx] DeMoD Acoustic FSK TX # [acoustic-tx] 2000/3000 Hz, 1200 baud, 48kHz SR # [acoustic-tx] frame #0 payload=00000000 # [acoustic-tx] frame #1 payload=00000001 # ... # You will hear the modem tones immediately. # They sound like a 1990s dial-up handshake. # That is correct.

## 5.4  Expected RX output

STDOUT — Machine B

# Machine B output after TX starts: [acoustic-rx] Waiting for mic AGC to settle... [acoustic-rx] Listening for frames... [rx #0000] type=DATA seq=0 payload=00000000 [rx #0001] type=DATA seq=1 payload=00000001 [rx #0002] type=DATA seq=2 payload=00000002 ... # Gaps in the sequence counter = dropped frames. # A few drops during the first 2-3 frames is normal (AGC settling). # Steady drops = too far away, or feedback loop, or buffer too small.

| i | WHAT IT SOUNDS LIKE At 1200 baud, mark=2000Hz, space=3000Hz, the transmitted audio is a warbling two-tone signal that shifts pitch 1200 times per second. Each JackFrame takes 53ms to transmit — you can hear individual frames as brief bursts. Sending continuously sounds like a sustained dial-up screech. |
| --- | --- |

# 6  Parameters

## 6.1  Modulator (acoustic_fsk_mod.dsp)

| PARAMETER | VALUE / NOTES |
| --- | --- |
| /acoustic_fsk_mod/f_mark | Mark tone Hz. Default 2000. Bit value = 1. |
| /acoustic_fsk_mod/f_space | Space tone Hz. Default 3000. Bit value = 0. |
| /acoustic_fsk_mod/symbol_rate | Baud rate. Default 1200. Must match RX. |
| /acoustic_fsk_mod/output_gain | Speaker volume 0..1. Default 0.7. Keep < 0.85 to avoid clipping. |

## 6.2  Demodulator (acoustic_fsk_demod.dsp)

| PARAMETER | VALUE / NOTES |
| --- | --- |
| /acoustic_fsk_demod/f_mark | Must exactly match TX f_mark. |
| /acoustic_fsk_demod/f_space | Must exactly match TX f_space. |
| /acoustic_fsk_demod/symbol_rate | Must exactly match TX symbol_rate. |
| /acoustic_fsk_demod/bpf_q | BPF Q factor. Default 3.5. Higher = narrower filters = better selectivity but slower. Try 5.0 in quiet rooms. |
| /acoustic_fsk_demod/output_gain | Soft symbol scale. Default 4.0. Increase if bits are marginal (soft values near 0). |

## 6.3  AcousticConfig (Haskell)

| PARAMETER | VALUE / NOTES |
| --- | --- |
| acSampleRate | 48000.0 Hz — universal. Change only if your interface requires it. |
| acSymbolRate | 1200.0 baud — standard. Try 600 for long range, 2400 for same-desk. |
| acFMark | 2000.0 Hz — safe for all laptop speakers. |
| acFSpace | 3000.0 Hz — safe for all laptop speakers. |
| acOutputGain | 0.7 — reasonable default. Reduce if speaker distorts. |
| acBpfQ | 3.5 — good for typical rooms. Increase in quiet environments. |
| acBlockSize | 256 samples — matches PipeWire default quantum. |
| acAcqFrames | 3 — frames discarded during mic AGC settle (~160ms). |
| acPreambleLen | 16 symbols — 13.3ms preamble before each frame. |
| acSilenceLen | 40 samples — one symbol of silence before preamble. |

# 7  Tuning & Troubleshooting

## 7.1  RX decodes nothing

| CAUSE | FIX |
| --- | --- |
| Speaker output muted or too quiet | Unmute system output. Check OS volume. Run speaker-test -t sine -f 2000 to verify 2kHz tone comes out. |
| Mic not capturing this device | Check pactl list sources — confirm correct mic is default source. Some laptops have array mics that notch 2-3kHz. |
| Feedback loop | Mute the RX machine system output entirely. Even quiet feedback saturates the demodulator. |
| Too far apart | Move to 30-40cm. Point speaker directly at mic. Angling matters. |
| Buffer overrun (xruns) | Increase JACK/PipeWire buffer to 512 samples. Check dmesg for RT scheduling issues. |
| bpf_q too high in noisy room | Reduce acBpfQ to 2.5. Wide filters are more tolerant of frequency drift. |

## 7.2  Test the signal without a second machine

TERMINAL — single-machine loopback test

# Loopback test on a single machine (requires separate output/input devices # OR virtual loopback via PipeWire): # Create a PipeWire null sink (virtual loopback): pactl load-module module-null-sink sink_name=loopback sink_properties=device.description=Loopback pactl load-module module-loopback source=loopback.monitor sink=loopback # Route acoustic-tx to the null sink: pactl set-default-sink loopback # Route acoustic-rx from the monitor source: pactl set-default-source loopback.monitor # Then run TX and RX in separate terminals on the same machine. # No speaker or mic required — pure digital loopback.

## 7.3  Verify tones with Audacity or baudline

TERMINAL

# Record a TX session to a WAV file: arecord -f cd -d 5 /tmp/fsk_capture.wav # 5 seconds, 44.1kHz stereo # Open in Audacity → Analyze → Plot Spectrum # Should see two peaks at 2000 Hz and 3000 Hz alternating. # Each peak should be 20-30 dB above the noise floor for reliable decode. # Or use baudline (included in the nix dev shell): baudline -stdin -samplerate 48000 -channels 1 -format s16 < /tmp/fsk_capture.wav

## 7.4  Adjusting tone frequencies

If your speaker has a notch at 2-3 kHz (common on thin laptop lids), shift both tones up: try f_mark=3000, f_space=4200. The deviation stays 1200 Hz, modulation index stays 1.0. Alternatively drop both tones down: f_mark=1200, f_space=2000 for speakers with better bass response. Update both TX and RX parameters to match.

# 8  The Jitter Argument

Acoustic FSK does not beat Ethernet on mean latency — one JackFrame takes 53ms to transmit through air. The interesting property is the predictability of that latency.

| PARAMETER | VALUE / NOTES |
| --- | --- |
| Acoustic FSK frame airtime | 53.3 ms (64 bits / 1200 baud) — fixed, deterministic |
| Acoustic FSK jitter | ±1 sample = ±0.021 ms @ 48kHz + ~±2ms multipath |
| Acoustic FSK total jitter | ±2–3 ms in a typical room |
| Ethernet UDP (quiet network) | ~0.3–1 ms mean, ±0.1–2 ms jitter |
| Ethernet UDP (under load) | ~1–5 ms mean, ±10–50 ms jitter |
| TCP (typical app) | ~1–10 ms mean, ±50–200 ms jitter under load |

The demo that makes this visible: run acoustic-rx and log the inter-frame arrival interval while playing a YouTube video on the RX machine. The acoustic FSK intervals will stay rock-steady at ~67ms (53ms frame + 13ms preamble). An equivalent UDP ping will start spiking as the browser and NIC compete for CPU and DMA bandwidth.

Haskell — interval logging

# Log inter-frame intervals on the RX side: # In AcousticRx.hs, print the timestamp difference between consecutive frames: import Data.Time.Clock (getCurrentTime, diffUTCTime) handler frame = do now <- getCurrentTime prev <- readIORef lastTime let interval_ms = realToFrac (diffUTCTime now prev) * 1000 :: Double printf "[rx] seq=%d interval=%.2fms\n" (jfSeq frame) interval_ms writeIORef lastTime now return True # Compare against: ping -i 0.067 -c 100 <other-machine-ip> ¦ grep time # Watch the ping jitter spike while the acoustic intervals stay flat.

That comparison — flat acoustic line vs spiking ping histogram — is the entire argument in one terminal window.

# 9  Quick Reference

## Key numbers

| PARAMETER | VALUE / NOTES |
| --- | --- |
| Sample rate | 48000 Hz (universal, every interface supports this) |
| Symbol rate | 1200 baud |
| Samples per symbol | 40 (48000 / 1200 — exact integer) |
| Mark frequency | 2000 Hz (bit = 1) |
| Space frequency | 3000 Hz (bit = 0) |
| Centre frequency | 2500 Hz |
| Deviation | 500 Hz (h = 0.833) |
| Carson bandwidth | 2200 Hz (1200 + 2×500) |
| Occupied band | 1400 – 3600 Hz |
| JackFrame bits | 64 (8 bytes) |
| Frame airtime | 53.3 ms (64 / 1200 × 1000) |
| Preamble | 13.3 ms (16 symbols) |
| Total TX / frame | ~67 ms including silence + preamble |
| Frames per second | ~15 (1200 / 80 symbols per frame) |
| JackFrame CRC pin | 0xC23F (DATA seq=0 payload=0xDEADBEEF) |
| Sync byte | 0xA5 |

## Shell commands

| PARAMETER | VALUE / NOTES |
| --- | --- |
| speaker-test -t sine -f 2000 | Verify 2kHz tone comes out of speaker before TX test. |
| arecord -f cd -d 3 test.wav | Record 3s of mic audio to check capture level. |
| sox test.wav -n stat | Check peak amplitude and RMS — should be -20 to -6 dBFS for good SNR. |
| pw-metadata -n settings | Check PipeWire quantum (buffer size). |
| pactl list sources short | List audio input devices. |
| pactl list sinks short | List audio output devices. |

DeMoD LLC  ·  info@demod.ltd  ·  GPL-3.0