DeMoD LLC

FAUST-SDR

DEMO GUIDE

RF transmission and reception of DCF transport frames using Haskell + Faust + SoapySDR

v0.3.0  ·  GPL-3.0  ·  info@demod.ltd

| TX PATH Haskell frame encoder → Faust BPSK DSP → SoapySDR TX stream | RX PATH SoapySDR RX stream → Faust Costas loop → Haskell frame decoder | FRAME 17 bytes, CRC-CCITT 9600 baud BPSK 14.1 ms/frame @ 2 MSPS |
| --- | --- | --- |

# 1  Hardware Requirements

Choose a hardware configuration that matches your demo setup. The simplest full-duplex demo uses a HackRF One with a loopback attenuator. The cheapest two-box demo uses two RTL-SDRs connected by coaxial cable.

| CONFIG | HARDWARE | TX / RX | NOTES |
| --- | --- | --- | --- |
| Recommended | HackRF One | Half-duplex (TX + RX via same antenna port) | Use 30–50 dB attenuator on loopback SMA cable. driver = "hackrf" |
| Budget | 2× RTL-SDR v3 | TX: demod-sdr-hs (HackRF or USRP required for real TX) RX: demod-rx-hs | RTL-SDR is RX-only. Pair with a HackRF TX or use the loopback pipe demo instead. |
| No hardware | Named pipe loopback | Full pipeline test in software only | sdr_transport.c plugin bridges TX and RX via POSIX FIFOs. No SoapySDR device needed. |
| Full-duplex | USRP B200 | Simultaneous TX + RX on separate antenna ports | driver = "uhd". Open withSdrDevice + withSdrRxDevice on the same physical device. |

| ! | RF SAFETY Always use a 30–50 dB inline SMA attenuator when looping TX directly into RX on the same device or coax cable. Transmitting into an unattenuated RX port at close range will damage the front-end LNA. |
| --- | --- |

# 2  Environment Setup

Both the C++ and Haskell dev shells are managed by Nix flakes. Enter one shell per terminal; the Faust compiler, SoapySDR headers, GHC, and cabal are all injected automatically.

## 2.1  Verify the SDR is detected

Plug in your hardware before entering the dev shell. The SoapySDR utility will enumerate every driver it finds:

TERMINAL — SDR probe

# Either shell includes sdr-probe as an alias for: SoapySDRUtil --probe # Expected output for HackRF: # [INFO] [HackRF] HackRF one found (serial=...) # driver: hackrf # hardware: HackRF One # # Expected output for RTL-SDR: # driver: rtlsdr # hardware: RTL2832U

## 2.2  Enter the Haskell shell

TERMINAL

cd haskell nix develop # full shell: GHC 9.6, cabal, Faust, SoapySDR, inspectrum nix develop .#headless # CI-friendly: GHC + cabal + Faust + SoapySDR only

## 2.3  Compile the Faust DSP

The Haskell cabal build compiles the C++ cbits (faust_bridge.cpp, soapy_bridge.cpp) but it does NOT run the Faust compiler automatically. You must generate the C++ from the .dsp file once before building, and again any time you modify a .dsp file.

TERMINAL — Faust compilation

# Inside haskell/ nix shell: # Compile the TX modulator (modulator_hs.dsp → build/modulator_gen.cpp): faust-build # Compile all TX + RX modem DSP files in one shot: faust-all-mod # modulator_hs, qpsk_mod, gmsk_mod, ask_mod, fsk_mod faust-all-demod # bpsk_demod, qpsk_demod, gmsk_demod, ask_demod, fsk_demod # Compile a specific file (e.g. BPSK demodulator): faust-bpsk-demod

| i | WHY THIS STEP faust_bridge.cpp #includes the generated C++ directly via <<includeclass>>. The generated file must exist before cabal build runs clang++ on the cbits. |
| --- | --- |

## 2.4  Build and test

TERMINAL

cabal-build # cabal build all (compiles library + demod-sdr-hs + demod-rx-hs) cabal-test # runs Frame + Symbol + Conformance test suites # Expected test output: # Frame + Symbol # Frame serialisation # encodeFrame produces exactly 17 bytes OK # round-trips through decodeFrame OK # QuickCheck: round-trip holds for arbitrary OK (+100 tests) # CRC properties # QuickCheck: any single-byte corruption ... OK (+100 tests) # Cross-language conformance # CRC pin: last two bytes are 0x42 0xDD OK # encodes reference frame to exact 17 bytes OK # 26 examples, 0 failures

# 3  Transmit Demo  (demod-sdr-hs)

demod-sdr-hs continuously encodes and transmits DCF beacon frames. Each frame carries a 32-bit big-endian counter in the 4-byte payload field, so the RX side can immediately count packet loss.

## 3.1  What happens end-to-end

| DeModFrame | → | encodeFrame 17 bytes | → | BPSK NRZ-L ±1 symbols | → | Faust RRC FIR 63-tap, α=0.35 | → | SoapySDR CF32 TX stream | → | RF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Each frame is 136 bits (17 bytes × 8). At 9600 baud and 2 MSPS sample rate, one frame is 28,288 samples — 14.1 ms of airtime. The RRC pulse shaper (63-tap Kaiser FIR, α=0.35) limits the occupied bandwidth to roughly 9600 × 1.35 = 12.96 kHz.

## 3.2  Run TX

TERMINAL — TX

# HackRF at 433.92 MHz, 2 MSPS, 20 dB TX gain: cabal run demod-sdr-hs -- \ --driver hackrf \ --freq 433.92e6 \ --rate 2e6 \ --gain 20 \ --count 100 # optional: stop after 100 frames (omit for infinite) # RTL-SDR is RX-only — this command will fail gracefully # with "could not open driver rtlsdr" because RTL-SDR has no TX. # Alias shorthand from inside the nix shell: cabal-run --driver hackrf --freq 433.92e6 --gain 20

Expected output:

STDOUT

[demod-hs] DeMoD Faust-SDR Haskell TX [demod-hs] driver=hackrf [demod-hs] freq=433.92 MHz [demod-hs] rate=2.0 MSPS [demod-hs] gain=20.0 dB [demod-hs] count=infinite [demod-hs] beacon #0 [demod-hs] beacon #1 [demod-hs] beacon #2 ...

## 3.3  TX parameters

| PARAMETER | VALUE / NOTES |
| --- | --- |
| --driver | SoapySDR driver string. hackrf ¦ uhd ¦ lime ¦ sdrplay |
| --freq | Centre frequency in Hz. 433.92e6 recommended (ISM 433 MHz band) |
| --rate | Sample rate in Hz. 2e6 = 2 MSPS. Must match RX. |
| --gain | TX gain in dB. HackRF: 0–47. USRP B200: 0–89. Start low. |
| --count | Stop after N frames. Omit for continuous TX. |

| i | FREQUENCY CHOICE 433.92 MHz is the center of the ISM 433 MHz license-exempt band. Keep TX power under the FCC Part 15 limit (-41.3 dBm/m at 3m) for §15.249 compliance. At 20 dB HackRF gain and a short SMA stub antenna, you are well under this limit at 1 m distance. |
| --- | --- |

# 4  Receive Demo  (demod-rx-hs)

demod-rx-hs runs the inverse pipeline: raw IQ from SoapySDR → Faust BPSK demodulator → soft-symbol slicer → DCF frame decoder → stdout.

## 4.1  What happens end-to-end

| RF | → | SoapySDR readCF32 4096 samp | → | Faust Costas loop + RRC MF | → | Soft slicer ÷ 208 sps | → | Frame sync scan + CRC | → | DeModFrame stdout |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The Costas loop in bpsk_demod.dsp requires a short lock-in period. The demodulator discards the first 4 frame-slots of samples (~57 ms at 2 MSPS) before attempting frame synchronisation. You will see "Acquiring carrier lock..." printed during this window.

## 4.2  Run RX

TERMINAL — RX

# RTL-SDR at 433.92 MHz, 2 MSPS, 40 dB RX gain: cabal run demod-rx-hs -- \ --driver rtlsdr \ --freq 433.92e6 \ --rate 2e6 \ --gain 40 \ --count 50 # stop after 50 frames (omit for infinite) # For HackRF loopback (TX + RX on same device via attenuator): # Run TX in one terminal, RX in a second terminal. # SoapySDR opens the same hackrf driver for both; the OS multiplexes. cabal run demod-rx-hs -- --driver hackrf --freq 433.92e6 --gain 30

Expected output:

STDOUT

[demod-rx] DeMoD Faust-SDR Haskell RX [demod-rx] driver=rtlsdr [demod-rx] freq=433.92 MHz [demod-rx] rate=2.0 MSPS [demod-rx] gain=40.0 dB [demod-rx] count=infinite [demod-rx] Acquiring carrier lock... [rx #0000] ver=1 type=BEACON seq=00001 src=0x0001 dst=0xFFFF payload=00000000 ts=0 [rx #0001] ver=1 type=BEACON seq=00002 src=0x0001 dst=0xFFFF payload=00000001 ts=14112 [rx #0002] ver=1 type=BEACON seq=00003 src=0x0001 dst=0xFFFF payload=00000002 ts=28224 ...

The payload field encodes a big-endian 32-bit counter (frame 0 = 0x00000000, frame 1 = 0x00000001, ...). Gaps in the counter show packet loss. The ts field is the 24-bit transmitter timestamp in microseconds.

## 4.3  RX parameters

| PARAMETER | VALUE / NOTES |
| --- | --- |
| --driver | rtlsdr ¦ hackrf ¦ uhd ¦ sdrplay |
| --freq | Must exactly match TX frequency. |
| --rate | Must exactly match TX sample rate (default 2e6). |
| --gain | RX gain in dB. RTL-SDR: 0–49.6. HackRF: 0–62. Start at 30–40. |
| --count | Stop after N decoded frames. |

# 5  No-Hardware Loopback Demo

The sdr_transport.c plugin bridges the full DCF → Faust-SDR → DCF pipeline using POSIX named pipes (FIFOs) instead of real RF hardware. This is the fastest way to verify end-to-end frame integrity without a radio.

## 5.1  Architecture

| DCF application (C SDK / Python / Lisp) | ⇄ | sdr_transport.so named pipe bridge | ⇄ | /tmp/dcf_sdr_tx /tmp/dcf_sdr_rx (POSIX FIFOs) | ⇄ | demod-sdr-hs demod-rx-hs |
| --- | --- | --- | --- | --- | --- | --- |

## 5.2  Steps

| 1 | Build the SDR transport plugin Compile sdr_transport.c as a shared library. Requires libcjson and pthreads. |
| --- | --- |

TERMINAL

cd dcf/c_sdk/plugins gcc -shared -fPIC -O2 \ -o libdcf_sdr_transport.so \ sdr_transport.c \ -lcjson -lpthread # Verify it exports the required entry point: nm -D libdcf_sdr_transport.so ¦ grep dcf_transport_create # Expected: 0000... T dcf_transport_create

| 2 | Create the named pipes mkfifo creates POSIX FIFOs. sdr_transport.c also calls mkfifo internally, so this is optional but useful to pre-verify permissions. |
| --- | --- |

TERMINAL

mkfifo /tmp/dcf_sdr_tx mkfifo /tmp/dcf_sdr_rx

| 3 | Start the RX side in terminal 1 demod-rx-hs reads 17-byte frames from /tmp/dcf_sdr_rx. It will block here until a writer opens the pipe. |
| --- | --- |

TERMINAL 1

# Terminal 1 — RX (reads from /tmp/dcf_sdr_rx) # We use "file" as the SoapySDR driver to redirect to the pipe. # Alternatively: cat /tmp/dcf_sdr_rx ¦ xxd -c 17 -g 1 # For a pure pipe loopback bypassing SoapySDR entirely: cat /tmp/dcf_sdr_tx ¦ python3 -c " import sys while True: b = sys.stdin.buffer.read(17) if not b: break sys.stdout.buffer.write(b) " > /tmp/dcf_sdr_rx & # Then run RX: cabal run demod-rx-hs -- --driver file --freq 0 --rate 2e6 --gain 0

| 4 | Start the TX side in terminal 2 demod-sdr-hs writes encoded frames. For the pipe demo, it writes directly to /tmp/dcf_sdr_tx. |
| --- | --- |

TERMINAL 2

# Terminal 2 — TX (writes to /tmp/dcf_sdr_tx) cabal run demod-sdr-hs -- \ --driver file \ --freq 0 \ --rate 2e6 \ --gain 0 \ --count 20

Both terminals should start exchanging frames within ~200 ms. Terminal 1 will print decoded frames with incrementing payload counters. Terminal 2 prints [demod-hs] beacon #N for each transmitted frame.

# 6  GNU Radio Companion Demo

The gr-demod OOT module provides dcf_frame_sink and dcf_frame_source as GNU Radio Python blocks. These integrate with any GRC flowgraph that produces or consumes a byte stream — pair them with GRC's built-in BPSK/QPSK/GFSK modulator blocks for a fully visual demo.

## 6.1  Install the OOT module

TERMINAL

cd gr-demod cmake -B build -DCMAKE_INSTALL_PREFIX=/usr/local cmake --build build sudo cmake --install build # Verify install: python3 -c "from demod import dcf_frame_sink, dcf_frame_source; print('OK')"

## 6.2  Run the pytest suite (no hardware required)

TERMINAL

python3 -m pytest gr-demod/python/demod/qa_dcf_frame.py -v # Expected output: # test_reference_vector PASSED (CRC pin 0x42DD asserted) # test_round_trip PASSED # test_all_types PASSED # test_bad_sync PASSED # test_crc_corruption PASSED # test_all_body_bytes_detected PASSED # test_single_frame_in_stream PASSED # test_two_frames_back_to_back PASSED # test_frame_with_leading_junk PASSED # 9 passed in 0.08s

## 6.3  Connect blocks in GRC

In a GRC flowgraph, wire the blocks as follows for a TX → RX loopback:

GRC FLOWGRAPH

[DeMoD DCF Frame Source] → [Pack K Bits (8→1)] → [BPSK Mod] (Python block) ¦ [Channel Model] ¦ [DeMoD DCF Frame Sink] ← [Unpack K Bits (1→8)] ← [BPSK Demod] (Python block) # Send a frame from Python (e.g. in a GRC variable block or Script Source): import pmt src = self.dcf_frame_source_0 msg = pmt.make_dict() msg = pmt.dict_add(msg, pmt.intern("payload"), pmt.init_u8vector(4, [0xDE, 0xAD, 0xBE, 0xEF])) msg = pmt.dict_add(msg, pmt.intern("type"), pmt.from_long(0)) # DATA src.to_basic_block()._post(pmt.intern("in"), msg)

## 6.4  Inspect received frames

dcf_frame_sink emits a PMT dictionary on its "frames" message port for every valid frame. Connect it to a Message Debug block to see frame fields in the GRC console, or wire to a custom message handler in Python:

PYTHON

# Fields in the PMT dict: # version → integer (0-15) # type → string "DATA" ¦ "ACK" ¦ "BEACON" ¦ "CTRL" # seq → integer (0-65535, rolling) # src_id → integer (0x0001 for this node) # dst_id → integer (0xFFFF = broadcast) # payload → u8vector(4) # timestamp_us → integer (24-bit, wraps at 16.7s) # raw → u8vector(17) ← full wire bytes # Example: extract payload in a message handler: def handle_frame(self, msg): if pmt.is_dict(msg): payload_pmt = pmt.dict_ref(msg, pmt.intern("payload"), pmt.PMT_NIL) payload = bytes(pmt.u8vector_elements(payload_pmt)) print(f"payload: {payload.hex().upper()}")

# 7  Cross-Language Conformance Verification

The CRC pin 0x42DD is the canonical test: if any language's codec produces a different CRC for the reference input, that codec has diverged from the wire format. Run all suites in one session to confirm end-to-end parity before a demo.

| PARAMETER | VALUE / NOTES |
| --- | --- |
| Language | Test command |
| Haskell | cd haskell && cabal-test |
| C/C++ | ctest --test-dir build -V (after cmake --build build) |
| Rust | cd dcf/rust && cargo test -- --nocapture |
| Go | cd dcf/go && go test -v -run TestCRCPin |
| Python/GR | python3 -m pytest gr-demod/python/demod/qa_dcf_frame.py -v |
| Lisp | sbcl --load hydramesh.lisp --eval "(run-tests)" --quit |

The reference vector all suites test against:

REFERENCE VECTOR

Input: version=1, DATA, seq=1, src=0x0001, dst=0xFFFF payload=DE AD BE EF, timestamp=0 Wire: D3 10 00 01 00 01 FF FF DE AD BE EF 00 00 00 42 DD ^^ ^^^^^ sync CRC pin

TERMINAL — CRC verification

# Quick CRC check from the shell (inside any nix shell): crc-check # Then paste: D3 10 00 01 00 01 FF FF DE AD BE EF 00 00 00 # Expected output: CRC = 42DD # Or one-liner: python3 -c " d=bytes.fromhex("D310000100 01FFFFDEAD BEEF000000".replace(" ","")) crc=0xFFFF for b in d: crc^=b<<8 for _ in range(8): crc=((crc<<1)^0x1021)if crc&0x8000 else crc<<1 print(hex(crc&0xFFFF)) # → 0x42dd "

# 8  Visualising the Signal

Capturing and inspecting the IQ stream is the fastest way to diagnose TX issues and tune the Costas loop for a live demo.

## 8.1  Capture to file with SoapySDRUtil

TERMINAL

# Capture 10 seconds of CF32 IQ at 433.92 MHz into a .cf32 file: SoapySDRUtil --rate=2e6 --freq=433.92e6 --gain=40 \ --direction=RX --driver=rtlsdr \ --args="driver=rtlsdr" --output=/tmp/capture.cf32 \ --duration=10

## 8.2  Open in inspectrum

TERMINAL

# Inside nix develop shell — iq-plot is an alias for inspectrum: iq-plot /tmp/capture.cf32 # In inspectrum: # 1. Set sample rate to 2000000 (2 MSPS) # 2. Adjust FFT size to 1024 or 2048 # 3. Drag the power range slider to make the signal visible # 4. Zoom into a burst to see individual RRC-shaped BPSK symbols

## 8.3  What you should see

| FEATURE | EXPECTED APPEARANCE |
| --- | --- |
| Occupied bandwidth | ~13 kHz wide (9600 × 1.35 rolloff). Tight rectangular shape — the 63-tap RRC FIR provides good spectral containment. |
| Symbol transitions | 208 samples per symbol at 2 MSPS. Each symbol should be ~5 pixels wide at 1024 FFT. |
| Frame bursts | 14.1 ms continuous blocks separated by negligible inter-frame gap. Counter in payload lets you confirm no frames are dropped. |
| BPSK constellation | Enable the constellation plugin (right-click → Add derived plot). Should show two tight clusters on the real axis at ±amplitude. Wide clusters indicate frequency offset not yet tracked by the Costas loop. |

# 9  Tuning & Troubleshooting

## 9.1  Costas loop does not acquire

Symptom: demod-rx-hs prints "Acquiring carrier lock..." but never decodes a frame, or decodes only garbage.

| CAUSE | FIX |
| --- | --- |
| Frequency offset > ±5 kHz | TX and RX --freq args must be identical. Cheap RTL-SDR dongles drift ±50 ppm; at 433 MHz that is ±21 kHz. Apply a TCXO, or use --freq offset correction (SoapySDRUtil --freq-correction=N). |
| loop_bw too narrow | Increase /bpsk_demod/loop_bw from 0.02 to 0.05. This widens acquisition range at the cost of noise. Set via setParam in runDemodulator or add a --loop-bw CLI flag. |
| RX gain too low | Costas loop needs SNR > ~6 dB to lock. Increase --gain. Check inspectrum — the signal should be clearly visible above the noise floor. |
| Symbol rate mismatch | TX and RX --rate must be identical. Verify both sides print the same MSPS value on startup. |
| demodAcqFrames too short | If the TX is far away, increase DemodConfig.demodAcqFrames from 4 to 8. This doubles the Costas lock-in window to ~113 ms. |

## 9.2  TX silently fails

TERMINAL

# Probe your hardware first: sdr-probe # If SoapySDRUtil --probe shows no devices: # RTL-SDR: unplug/replug, check lsusb (idVendor=0bda) # HackRF: hackrf_info (if not found: try different USB3 port) # All: check SoapySDR module is installed for your driver SoapySDRUtil --find

## 9.3  Faust compile fails

TERMINAL

# "cannot open DSP file" — run from inside haskell/ directory: cd haskell && faust-build # "unknown identifier" in generated C++ — update Faust libraries: # The .dsp files use stdfaust.lib, filters.lib, oscillators.lib, maths.lib. # Verify FAUST_ARCH_PATH is set: echo $FAUST_ARCH_PATH # should print <nix-store>/share/faust # Re-enter the nix shell if the var is missing: nix develop

## 9.4  CRC conformance test fails

TERMINAL

# Run the cross-language pin check: python3 -c " d=bytes.fromhex("D3100001000 1FFFFDEADBEef000000".replace(" ","")) crc=0xFFFF for b in d: crc^=b<<8 for _ in range(8): crc=((crc<<1)^0x1021)if crc&0x8000 else crc<<1 crc&=0xFFFF assert crc==0x42DD, f"FAIL: got {hex(crc)}" print("PASS: 0x42DD") "

# 10  Quick Reference

## Shell aliases (inside nix develop)

| PARAMETER | VALUE / NOTES |
| --- | --- |
| faust-build | Compile modulator_hs.dsp → build/modulator_gen.cpp |
| faust-all-mod | Compile all 5 modulator DSP files |
| faust-all-demod | Compile all 5 demodulator DSP files |
| cabal-build | cabal build all (library + demod-sdr-hs + demod-rx-hs) |
| cabal-test | Run frame + conformance test suite |
| cabal-run | cabal run demod-sdr-hs -- (TX with default args) |
| sdr-probe | SoapySDRUtil --probe (enumerate devices) |
| iq-plot <file> | Open .cf32 IQ file in inspectrum |
| frame-rx | xxd -c 17 -g 1 (hex-dump raw frames from stdin) |
| crc-check | Compute CRC-CCITT over 15 bytes from stdin |
| frame-tx-test | Encode reference frame and verify 0x42DD CRC pin |

## Key numbers

| PARAMETER | VALUE / NOTES |
| --- | --- |
| Symbol rate | 9600 baud (default) |
| Sample rate | 2,000,000 MSPS (default, must match TX/RX) |
| Samples per symbol | 208 (2e6 / 9600) |
| Frame length | 136 bits (17 bytes × 8) |
| Samples per frame | 28,288 (136 × 208) |
| Frame airtime | 14.1 ms (28288 / 2e6) |
| Acquisition window | ~57 ms (4 frames × 14.1 ms) |
| Occupied bandwidth | ~12.96 kHz (9600 × (1 + 0.35)) |
| RRC filter taps | 63-tap Kaiser, α=0.35, β=8.0 |
| CRC algorithm | CRC-CCITT poly 0x1021, init 0xFFFF |
| Reference CRC pin | 0x42DD (over D3...00 reference input) |
| Sync byte | 0xD3 |
| Broadcast dst_id | 0xFFFF |

DeMoD LLC  ·  info@demod.ltd  ·  GPL-3.0